DESIGN AND CONFIGURATION OF A SIMPLE NETWORK TOPOLOGY USING ONE ROUTER, TWO SWITCHES, AND PCS IN CISCO PACKET TRACER

EXPT: 10

AIM:
To design and configure a basic network topology consisting of one router, two switches, and multiple PCs in Cisco Packet Tracer, and to verify successful communication between the connected networks.

INTRODUCTION:
In computer networking, routers are used to connect different networks, while switches connect multiple devices within the same local area network (LAN).
In this experiment, two LANs—each connected through a switch—are interconnected via a router. Each LAN will be assigned a unique IP address range, and the router will enable packet transmission between them.

ALGORITHM:

- Open Cisco Packet Tracer.

- Select and place the following network devices:

  - 1 Router (e.g., Cisco 2911)

  - 2 Switches (e.g., Cisco 2960)

  - 4 PCs

- Connect the devices using Copper Straight-Through cables as follows:

  - PC0 → Switch0 (FastEthernet0/1)

  - PC1 → Switch0 (FastEthernet0/2)

  - PC2 → Switch1 (FastEthernet0/1)

  - PC3 → Switch1 (FastEthernet0/2)

  - Switch0 → Router (GigabitEthernet0/0)

  - Switch1 → Router (GigabitEthernet0/1)

- Assign IP addresses to the PCs:

  - LAN 1 (Switch0 side): 192.168.1.0/24 → PC0, PC1

  - LAN 2 (Switch1 side): 192.168.2.0/24 → PC2, PC3

- Configure the router interfaces:

  - Interface GigabitEthernet0/0: IP 192.168.1.1 255.255.255.0

  - Interface GigabitEthernet0/1: IP 192.168.2.1 255.255.255.0

  - Use the command no shutdown to activate both interfaces.

- Set the Default Gateway on each PC:

  - For LAN1 PCs → 192.168.1.1

  - For LAN2 PCs → 192.168.2.1

- Verify the configuration:

  - Use the ping command from a PC in LAN1 to a PC in LAN2.

  - Check for successful reply messages.

- If packets are received successfully, the network setup is verified and functional.

NETWORK TOPOLOGY:

OUTPUT:

RESULT:
A simple network topology using one router, two switches, and multiple PCs was successfully designed and configured in Cisco Packet Tracer. Communication between both LANs was verified through the ping command.